SECTION 07710

EDGE METAL ROOF SPECIALTIES

- GENERAL

  - SECTION INCLUDES

    - SecurEdge 400 Drip Edge.

    - SecurEdge 400 Econo Coping.

    - SecurEdge 400 Fascia Cover.

    - SecurEdge 400 Gravel Stop.

    - SecurEdge 400 Gutter.

    - SecurEdge 400 Snap Gravel Stop.

    - SecurEdge 400 Spring-Tite Coping.

    - Accessories.

  - RELATED SECTIONS

    - Section 06150 - Wood Decking.

    - Section 06160 - Sheathing: Plywood and oriented strand board.

    - Section 06200 - Wood Trim.

    - Section 07220 - Roof and Deck Insulation: Roof board insulation.

    - Section 07310 - Roof Shingles.

    - Section 07320 - Roof Tiles.

    - Section 07410 - Metal Roof and Wall Panels: Preformed metal roofing.

    - Section 07500 - Membrane Roofing.

    - Section 07620 - Sheet Metal Flashing and Trim.

    - Section 07720 - Roof Accessories: Soffit vents, roof pavers, etc.

  - REFERENCES

    - ANSI/SPRI ES-1 - Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

    - ANSI/SPRI GD-1 – Structural Design Standard for Gutter Systems Used with Low Slope Roofing Systems.

    - SPRI Single Ply Roofing Industry Standards.

  - PERFORMANCE REQUIREMENTS

    - ANSI/SPRI ES-1 or GD-1 tested and certified by a 3rd party testing laboratory active in the testing of industry standard edge metal systems.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - Product Data: Manufacturer’s data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings: Show profiles, joining method, location of accessory items, attachment and flashing details, membrane roof interface, and dimensions.

    - Finish/Color Samples: For each color product specified, two complete sets of color charts representing Manufacturer’s full range of minimum 35 colors and patterns.

    - Verification Samples: For each color product specified, two sample chips representing actual product, color, and patterns.

    - Manufacturer’s Certificates: Certify products meet specified performance requirements.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Company certified by Carlisle Metal Products as a “Authorized Fabricator” in manufacturing Edge Metal Products specified in this section.

    - 3rd Party QA Certification: “Authorized Fabricator” certified and inspected by 3rd party ANSI/SPRI authorized testing laboratory for quality assurance. Current annual certificate required.

    - Installer Qualifications: Company certified by Carlisle Metal Products as a “Certified Installer” in the installation of Edge Metal Products specified in this section.

    - Mock-Up: Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

      - Finish areas designated by Architect.

      - Do not proceed with remaining work until workmanship, color, and sheen are approved by Architect.

      - Refinish mock-up area as required to produce acceptable work.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products in Manufacturer’s unopened packaging until ready for installation.

    - Store materials in a dry, protected, well-vented area.

    - Remove protective plastic surface film immediately before installation.

    - Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

  - SEQUENCING

    - Ensure that information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

    - Coordinate installation with roof membrane manufacturer's installation instructions.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by Manufacturer for optimum results. Do not install products under environmental conditions outside Manufacturer's absolute limits.

  - WARRANTY

    - Bare/Unpainted Corrosion Warranty: Acrylic Coated Galvalume AZ-55 comes with a 25 year, 6 month warranty (salt water locations within 2,800 feet from direct surf, 1,200 feet from a large bay, or 600 feet from marshes and canals are excluded).

    - Paint Finish Warranty: Carlisle Metal Products 35 year “Non Pro-Rated” Paint Finish Waranty for all colors. Carlisle Metal Products standard form in which manufacturer agrees to repair finish or replace material that show evidence of deterioration of factory-applied finishes within specified warranty period.

  - Fluoropolymer Paint Finish Warranty Period: 35 years from date of substantial completion.

  - Failures include, but are not limited to, the following:

    - Structural failures including rupturing, cracking, or puncturing.

  - Deterioration of metals, metal finishes, and other materials beyond normal weathering and excluding salt water locations within specified number of feet

    - Carlisle Metal Products Job Information Sheet and Warranty Applications must be completed prior to delivery of the warranty.

  - Energy Efficient Roof Coatings: A number of Carlisle Metal Products Standard Colors meet the minimum standard energy efficient requirements as outlined in the Energy Star Program.

    - Galvalume Substrate Warranty AZ-50: 20 year (salt water locations within 2,800 feet from direct surf, 1,200 feet from a large bay, or 600 feet from marshes and canals are excluded).

    - Aluminum Substrate Warranty: 25 year within 2800 feet of coastline / 30 year greater than 2800 feet of coastline.

PART   2  PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Carlisle Metal Products, Toll Free Tel: 800-479-6832 ; Fax: 717-245-7053 ; Web: www.carlislemetalproducts.com

    - Substitutions: Not permitted.

  - ROOF EDGE PRODUCTS

    - SecurEdge 400 Drip Edge: Decorative metal Drip Edge with continuous 22 Gauge Cleat. Cleat to be pre-punched by Carlisle Metal Products for precise attachment.

      - Construction:

        - SecurEdge 400 Drip Edge Material:

          - .040 inch (1 mm) thick formed aluminum.

          - 24 gauge Galvalume coated steel.

        - SecurEdge 400 Drip Edge Finish:

          - 35 year color as selected by the Architect from Carlisle Metal Product’s color chart.

          - Galvaume (Acrylic Coated Galvalume).

          - Bare/Unpainted mill finish aluminum.

        - SecurEdge 400 Drip Edge Length:

          - Standard 10’-0”(3.048m), or up to 20’-0”(6.096m) maximum.

        - SecurEdge 400 Drip Edge Attachment:

          - 1-1/4” Stainless Steel Ring Shank Nails provided by Carlisle Metal Products attached to substrate at 12” on center through Cleat.

          - Certified per ANSI/SPRI ES-1 Standard to a design pressure of 390 lbs./ft2 (24GA Steel) and 410 lbs./ft2 (.040” AL) to comply with the International Building Code.

    - SecurEdge 400 Econo Coping: Decorative metal Econo Coping with continuous 22 Gauge Cleat. Cleat to be pre-punched by Carlisle Metal Products for precise attachment.

      - Construction:

        - SecurEdge 400 Econo Coping Material:

          - .040 inch (1 mm) thick formed aluminum.

          - 24 gauge Galvalume coated steel.

        - SecurEdge 400 Econo Coping Finish:

          - 35 year color as selected by the Architect from Carlisle Metal Product’s color chart.

          - Galvalume (Acrylic Coated Galvalume).

          - Bare/Unpainted mill finish aluminum.

        - SecurEdge 400 Econo Coping Length:

          - Standard 10’-0”(3.048m), or up to 20’-0”(6.096m) maximum.

        - SecurEdge 400 Econo Coping Attachment:

          - 1-1/4” Stainless Steel Ring Shank Nails provided by Carlisle Metal Products attached to substrate face side at 12” on center through Cleat.

          - #9 Washered Drexmet Woodscrew provided by Carlisle Metal Products attached to substrate roof side at 12” on center.

          - Certified 16” maximum width per ANSI/SPRI ES-1 Standard to a design pressure of 250 lbs./ft2 (24GA Steel) and 260 lbs./ft2 (.040” AL) to comply with the International Building Code.

          - Certified 20” maximum width per ANSI/SPRI ES-1 Standard to a design pressure of 210 lbs./ft2 (24GA Steel) and 210 lbs./ft2 (.040” AL) to comply with the International Building Code.

    - SecurEdge 400 Fascia Cover: Decorative metal Fascia Cover with continuous 22 Gauge Cleat. Cleat to be pre-punched by Carlisle Metal Products for precise attachment.

      - Construction:

        - SecurEdge 400 Fascia Cover Material:

          - .040 inch (1 mm) thick formed aluminum.

          - 24 gauge Galvalume coated steel.

        - SecurEdge 400 Fascia Cover Finish:

          - 35 year color as selected by the Architect from Carlisle Metal Product’s color chart.

          - Galvalume (Acrylic Coated Galvalume).

          - Bare/Unpainted mill finish aluminum.

        - SecurEdge 400 Fascia Cover Length:

          - Standard 10’-0”(3.048m), or up to 20’-0”(6.096m) maximum.

        - SecurEdge 400 Fascia Cover Attachment:

          - #10 Pancake Head Woodscrew Stainless Steel provided by Carlisle Metal Products attached to substrate at 12” on center through Cleat.

          - Certified per ANSI/SPRI ES-1 Standard to a design pressure of 135 lbs./ft2 (24GA Steel) and 135 lbs./ft2 (.040” AL) to comply with the International Building Code.

    - SecurEdge 400 Gravel Stop: Decorative metal Gravel Stop with continuous 22 Gauge Cleat. Cleat to be pre-punched by Carlisle Metal Products for precise attachment.

      - Construction:

        - SecurEdge 400 Gravel Stop Material:

          - .040 inch (1 mm) thick formed aluminum.

          - 24 gauge Galvalume coated steel.

        - SecurEdge 400 Gravel Stop Finish:

          - 35 year color as selected by the Architect from Carlisle Metal Product’s color chart.

          - Galvalume (Acrylic Coated Galvalume).

          - Bare/Unpainted mill finish aluminum.

        - SecurEdge 400 Gravel Stop Length:

          - Standard 10’-0”(3.048m), or up to 20’-0”(6.096m) maximum.

        - SecurEdge 400 Gravel Stop Attachment:

          - 1-1/4” Stainless Steel Ring Shank Nails provided by Carlisle Metal Products attached to substrate at 12” on center through Cleat.

          - Certified per ANSI/SPRI ES-1 Standard to a design pressure of 180 lbs./ft2 (24GA Steel) and 170 lbs./ft2 (.040” AL) to comply with the International Building Code.

    - SecurEdge 400 Gutter: Decorative metal Gutter.

      - Construction:

        - SecurEdge 400 Gutter Material:

          - .040 inch (1 mm) thick formed aluminum.

          - 24 gauge Galvalume coated steel.

        - SecurEdge 400 Gutter Finish:

          - 35 year color as selected by the Architect from Carlisle Metal Product’s color chart.

          - Galvalume (Acrylic Coated Galvalume).

          - Bare/Unpainted mill finish aluminum.

        - SecurEdge 400 Gutter Length:

          - Standard 10’-0”(3.048m), or up to 20’-0”(6.096m) maximum.

        - SecurEdge 400 Gutter Attachment:

          - #10 Pancake Head Woodscrew Stainless Steel provided by Carlisle Metal Products attached to substrate at 12” on center.

          - Carlisle Metal Products Gutter Bracket made from .063” aluminum spaced to meet performance requirements and design loads.(24” o.c. max.)

          - #9 Washered Woodscrew Stainless Steel provided by Carlisle Metal Products attached through gutter bracket to substrate.

          - Certified per ANSI/SPRI GD-1 Standard to a design pressure of below to comply with the International Building Code.		24GA Steel Gutter			.040” Aluminum Gutter		G-1 Horizontal = 200psf		G-1 Horizontal = 210psf		G-2 Vertical = 90psf		G-2 Vertical = 90psf		G-3 Static = 280psf			G-3 Static = 290psf

    - SecurEdge 400 Snap Gravel Stop: Decorative metal Snap Gravel Stop with continuous 22 Gauge Cleat. Cleat to be pre-punched by Carlisle Metal Products for precise attachment.

      - Construction:

        - SecurEdge 400 Snap Gravel Stop Material:

          - .040 inch (1 mm) thick formed aluminum.

          - 24 gauge Galvalume coated steel.

        - SecurEdge 400 Snap Gravel Stop Finish:

          - 35 year color as selected by the Architect from Carlisle Metal Product’s color chart.

          - Galvalume (Acrylic Coated Galvalume).

          - Bare/Unpainted mill finish aluminum.

        - SecurEdge 400 Snap Gravel Stop Length:

          - Standard 10’-0”(3.048m), or up to 20’-0”(6.096m) maximum.

        - SecurEdge 400 Snap Gravel Stop Attachment:

          - 1-1/4” Stainless Steel Ring Shank Nails provided by Carlisle Metal Products attached to substrate at 12” on center through Cleat.

          - Certified per ANSI/SPRI ES-1 Standard to a design pressure of 90 lbs./ft2 (24GA Steel) and 75 lbs./ft2 (.040” AL) to comply with the International Building Code.

    - SecurEdge 400 Spring-Tite Coping: Decorative metal Spring-Tite Coping with continuous 22 Gauge Cleat. Cleat to be pre-punched by Carlisle Metal Products for precise attachment.

      - Construction:

        - SecurEdge 400 Spring-Tite Coping Material:

          - .040 inch (1 mm) thick formed aluminum.

          - 24 gauge Galvalume coated steel.

        - SecurEdge 400 Spring-Tite Coping Finish:

          - 35 year color as selected by the Architect from Carlisle Metal Product’s color chart.

          - Galvalume (Acrylic Coated Galvalume).

          - Bare/Unpainted mill finish aluminum.

        - SecurEdge 400 Spring-Tite Coping Length:

          - Standard 10’-0”(3.048m), or up to 20’-0”(6.096m) maximum.

        - SecurEdge 400 Spring-Tite Coping Attachment:

          - #10 Pancake Head Woodscrew Stainless Steel provided by Carlisle Metal Products attached to substrate at 12” on center through Cleat.

          - Certified 16” maximum width per ANSI/SPRI ES-1 Standard to a design pressure of 105 lbs./ft2 (24GA Steel) and 90 lbs./ft2 (.040” AL) to comply with the International Building Code.

          - Certified 20” maximum width per ANSI/SPRI ES-1 Standard to a design pressure of 90 lbs./ft2 (24GA Steel) and 75 lbs./ft2 (.040” AL) to comply with the International Building Code.

  - ACCESSORlES

    - Miters, Termination Ends, and End Caps shall be fabricated by Carlisle Metal Products to suit the conditions indicated on the Shop Drawings.

    - Provide Carlisle Metal Products fasteners consistent with manufacturer's instructions for each product that is suitable for the substrate to which it is being installed.

PART  3  EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - Verify that the substrate is dry, clean and free of foreign matter.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the Manufacturer for achieving the best result for the substrate under the project conditions.

    - Verify Carlisle Metal Products's roof edge details for application to fit the assembly prior to fabrication.

  - INSTALLATION

    - Install in accordance with Carlisle Metal Products's installation instructions.

    - Use provided fasteners consistent with Carlisle Metal Products's instructions.

    - Install water cut-off mastics and sealants, as recommended by Carlisle.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up scratched finish areas with Carlisle Metal Products “Touch-Up Paint Pen”.

    - Repair or replace damaged products before Substantial Completion.

END OF SECTION